锐捷5750交换机与蓝海卓越认证计费

对接使用手册

Portal认证

目   录

蓝海卓越认证计费设置	3

一、 登录系统	3

二、 对接NAS或BRAS设置	4

第三方对接的必要条件	4

RADIUS对接	5

PORTAL对接	5

三、 编辑模板	6

四、 设定模板推送策略	7

默认模板	7

模板策略	8

五、 认证方式说明	9

六、 用户添加及管理	10

第一步：用户组管理	10

第二步：套餐设定	11

第三步：用户管理	12

锐捷交换机配置参考	14

七、 在锐捷交换机上配置PORTAL重定向设置	14

1 配置文件参考	14

# 蓝海卓越认证计费设置

## 登录系统

本系统是基于CENTOS安装，安装完成，在电脑上配置同网段的IP地址，在电脑浏浏览器中使用该IP地址即可访问，默认WEB端口：7788.

示例如下：

http://192.168.1.1:7788

登录界面如下图：

默认使用的帐号密码为：

用户名：admin

密码：admin123或Admin@123

登录成功界面如下图：

## 对接NAS或BRAS设置

本系统通常是与第三方厂家的NAS设备对接，可以是AC/BRAS/交换机/路由/防火墙等

### 第三方对接的必要条件

- 需要支持某一种PORTAL协议以及标准的RADIUS认证协议。

- 终端在未认证的情况下可以直接访问Portal认证页面，即打开浏览器输入：http://192.168.1.1:9090（示例，IP地址自行修改），通常要求NAS设备上开启白名单功能，将PORTAL认证页面放入白名单中。

- NAS设备可以同蓝海卓越统一认证服务器之间进行通信，NAS与蓝海卓越统一认证管理系统能通过IP地址相互通讯及访问，如果是远程访问，需要保证相关的端口不被防火墙屏蔽。标准常用端口如下：

- RADIUS端口：1812、1813、3799

- WEB端口：80、8888、8080、9090（WEB端口均可以自定义，其中默认的PORTAL展示端口为9090）

- PORTAL端口：2000

本系统上的对接设置为为两块，RADIUS对接和PORTAL对接，分别如下：

### RADIUS对接

进入主系统—服务设置—NAS对接，点击左上角的“添加”按纽，显示如下图：

根据实际对接情况填写：

和PANABIT对接，信息如下：

- 名称：自定义方便识别的名字

- IP地址：PANABIT设备与计费通讯的IP地址

- 共享秘钥：双方对接使用的共享秘钥

- COA端口：3799

### PORTAL对接

进入主系统—服务设置—PORTAL对接配置，点击左上角的“添加”按纽，显示如下图：

根据实际对接情况填写：

和PANABIT对接，信息如下：

- IP地址：PANABIT设备与计费通讯的IP地址

- 共享秘钥：双方对接使用的共享秘钥

- 设备名称：通常使用IP地址即可

- 协议版本号：1

- 认证方式：PAP

- 通讯端口：2000

- 超时时长：3秒以上，不超过10秒

## 编辑模板

编辑模板通过打开主页面—认证管理—模板管理页面进入：

每个模板底部有色块，其中蓝色表示未使用模板 ，桔色表示被引用模板，红色表示默认模板。

鼠标悬浮在模板上方，可以对模板进行相应的操作：

## 设定模板推送策略

### 默认模板

参考上一切的编辑模板中，如需设置默认模板，在模板悬浮窗口点击：设为默认，即可将该模板设置为默认模板。参考下图。

默认模板只能是全局唯一。

设置了模板策略后，必须点击：刷新按纽，以刷新模板策略。

### 模板策略

本系统可以根据指定的时间、SSID、APMAC、ACIP等不同的参数匹配，推送不同的模板。

进入主页面—认证管理页面，关于模板策略的页面有如下几个：

- 认证策略：策略设定页面

- AP信息：AP信息录入管理页面，主要是AP的MAC地址和备注，可以使用页面添加和批量导入。

- AP组管理：对AP进行组设置，方便策略设定

- 时间管理：设定时间条件，方便策略组合。

- SSID管理：设定SSID名称，方便策略组合。

认证策略设置

在本页面中，除了名称和描述性的字段个，和策略有关的字段共如下几个：

- 认证模板

- AP组名称

- SSID名称

- 时间策略

通过这几种因素的组合，可以设定不同的PORTAL推送策略

## 认证方式说明

本系统支持如下认证方式：

- 一键认证

- 帐号密码认证

- 短信认证

- 微信连WIFI与强关注

- AD域（LDAP）认证

- 访客扫码认证

- APP认证

- 二次认证

- MAC二次免认证

- MAC无感知流程

- 哑终端认证

## 用户添加及管理

### 第一步：用户组管理

用户组属于管理范围，和用户的认证属性无关，在本系统中，用户组可以做自定义，多级扩展，并且组的名称可以由管理员自定义，管理起来比较灵活。

在初始使用本系统时，应该先根据需要建好用户组，在用户级中再进行添加用户操作。具体操作如下：

主界面—AAA—用户组管理—右上方的添加用户组版块—进行添加用户组操作

在与PANABIT对接时，厂商需选择：PANABIT，如下图：

- 用户组名称：管理员定义的用户组名称

- 选择厂商：此处选择厂商，决定了该组的用户建立后生成的对应属性，请严格按照对应厂商选择，否则可能会出现无法限速，无法认证等情况

- 记帐包发送时间：要求NAS发送记帐包的间隔时间，单位为秒，通常建议5000用户以内设置为300秒，5000用户以上设置为1200秒。

- MAC绑定数：设置该组默认的MAC绑定数，用于开户时自动生成该属性。

- 用户在线数：设置该组默认的MAC绑定数，用于开户时自动生成该属性。

- 选择套餐：选择可以在该用户组使用的套餐，该选择也可以在建立套餐时反选用户组。

- 套餐介绍：用于展示在用户登录自助系统后，查看套餐介绍。

- 公告内容：用于在用户登录自助系统后，查看公司内容的设置。

- 删除当前用户组：如果当前用户组下有用户，或下级用户组有用户，则不可被删除。

### 第二步：套餐设定

带宽组

设定套餐前，我们需要先设定带宽组，合理的带宽分配，帮让用户体验更好。

点击主菜单—AAA—带宽组—添加带宽组，显示如下界面

### 第三步：用户管理

用户开户

用户开户有三种方式

- 管理员前台页面开户

- 接口开户

- 用户自助开户

这里描述的是第一种，管理员前台开户

操作步骤：

主界面—AAA—用户列表—选择用户组—添加用户，出现如下界面

- 用户组：点击添加用户前，就已经选择好了用户组，因此这里的用户组不可以设置。

- 选择套餐：可以通过下拉列表显示可用套餐，这里的套餐只显示该用户组可用套餐。选择了套餐后，下方的预存金额框里，会自动填充该套餐的金额。如无特殊原因，通常无需改动

- 帐号：用户上网认证所用的帐号

- 密码：用户上网认证所用的密码

- 姓名 / 身份证号码：用户真实姓名和用户身份证号码，该填写项目可以对用户进行身份证与姓名对应的实名认证。(该功能是否生效，取决于设置，设置地址为：主界面—系统管理—开户配置—是否进行实名认证，如果开启，则在开户提交时，会强制调用接口进行实名认证，身份证号和姓名不符的，无法进行开户)

- 手机号码、邮箱、住址：均为用户信息，根据实际情况填写即可

- 预存金额：选择套餐后，会自动生成，通常无需填写

- 计时类型：分为立即计时和上线计时，立即计时表示开户完成后，即以开户时间为用户的开始计时时间，并核算出用户的结束时间。上线计时表示，用户会在第一次上线后，才开始计算用户的开始时间，如果用户未上线认证过，则用户的上线时间和下线时间均为空。

- MAC绑定数量：如果填0，则为不绑定，用户每次上线，均会记录最新上线的设备的MAC地址。如果绑定，则绑定填写的数字，系统会自动绑定，无需人工干预。

- 允许同时在线人数：同一个帐号允许同时在线人数

- 该参数和MAC绑定数量是相互制约的关系。

- 如绑定5个MAC，但是允许同时在线人数是2，则在这5个绑定的终端中，用户任意登录，但只会保持两台终端在线，后上线的会把前面上线的用户踢掉。(该功能是否生效，取决于设置，设置地址为：主界面—系统管理—开户配置—超过最大在线用户是否挤用户下线)

- 如绑定2个MAC，但是允许同时在线人数是5，则只有这两个绑定了MAC的终端可以上线，其他未绑定MAC的终端无法上线。

- 绑定APID/APMAC / 绑定NASIP / 绑定SSID

这三种绑定的含义如字面意义，如果要绑定，只需要开户时勾选，则用户第一次上线后，会自动进行绑定。

# 锐捷交换机配置参考

## 在锐捷交换机上配置PORTAL重定向设置

### 1 配置文件参考1（5750E)

NBS5750-E#sh run

Building configuration...

Current configuration: 6440 bytes

version NBS5750_RGOS 11.4(1)B70

hostname NBS5750-E

!

errdisable recovery interval 300

no spanning-tree

!

ip access-list extended te

10 permit icmp host 114.114.114.114 host 172.16.10.1

20 permit icmp host 172.16.10.1 host 114.114.114.114

30 permit ip any any

!

ip access-list counter te

!

ip portal source-interface AggregatePort 1

http redirect direct-site 10.10.10.254 arp

http redirect direct-site 10.10.11.2 arp

http redirect direct-site 172.16.1.1 arp

http redirect direct-site 172.16.10.1 arp

http redirect direct-site 172.16.10.189

http redirect direct-site 172.16.20.1 arp

web-auth direct-host 172.16.1.0 255.255.255.0

web-auth direct-host 10.10.11.2

web-auth direct-host 10.10.10.0 255.255.255.0

web-auth direct-host 172.16.10.1 arp

!

web-auth template eportalv2

ip 10.10.11.2

port 2000

url http://10.10.11.2:9090

authentication default

accounting default

!

web-auth template eportalv1

!

username admin password ruijie123

!

aaa new-model

!

aaa accounting update periodic 15

aaa accounting update

aaa accounting network default start-stop group radius

aaa authentication dot1x default group radius

aaa authentication web-auth default group radius

!

cwmp

acs url http://118.190.126.198/service/acs/G1QT1Z3005423

cpe inform interval 180

!

service dhcp

!

ip dhcp pool AP

lease 0 4 0

network 172.16.1.0 255.255.255.0

dns-server 222.222.222.222 223.5.5.5

default-router 172.16.1.1

!

ip dhcp pool sta2

lease 0 4 0

network 172.16.20.0 255.255.255.0

dns-server 222.222.222.222 223.5.5.5

default-router 172.16.20.1

!

ip dhcp pool sta1

lease 0 4 0

network 172.16.8.0 255.255.252.0

dns-server 222.222.222.222 223.5.5.5

default-router 172.16.10.1

!

install 0 NBS5750-28GT4XS-E

!

sysmac c0b8.e6da.dce4

ip name-server 223.5.5.5

ip name-server oob 223.5.5.5

ip name-server 114.114.114.114

ip name-server oob 114.114.114.114

!

enable service web-server http

enable service web-server https

webmaster level 0 username admin password ruijie123

mmu buffer-mode normal

!

nfpp

no arp-guard enable

no icmp-guard enable

no ip-guard enable

no dhcp-guard enable

no dhcpv6-guard enable

no nd-guard enable

!

no service password-encryption

!

ip radius source-interface AggregatePort 1

radius-server host 10.10.11.2 key testing123

!

redundancy

!

clock timezone AsiaShanghai +8 0

!

no zam

enable password ruijie123

!

vlan range 1,10,20

!

interface GigabitEthernet 0/1

no switchport

!

interface GigabitEthernet 0/2

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/3

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/4

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/5

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/6

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/7

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/8

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/9

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/10

switchport mode trunk

rldp port loop-detect shutdown-port

anti-arp-spoofing ip 172.16.10.1

web-auth enable eportalv2

!

interface GigabitEthernet 0/11

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/12

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface GigabitEthernet 0/13

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/14

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/15

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/16

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/17

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/18

switchport access vlan 10

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/19

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/20

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/21

port-group 10

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/22

port-group 10

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/23

port-group 10

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/24

port-group 10

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/25

switchport mode trunk

rldp port loop-detect shutdown-port

!

interface GigabitEthernet 0/26

no switchport

port-group 1

!

interface GigabitEthernet 0/27

no switchport

port-group 1

!

interface GigabitEthernet 0/28

no switchport

port-group 1

!

interface TenGigabitEthernet 0/29

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface TenGigabitEthernet 0/30

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface TenGigabitEthernet 0/31

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface TenGigabitEthernet 0/32

switchport mode trunk

rldp port loop-detect shutdown-port

web-auth enable eportalv2

!

interface AggregatePort 1

no switchport

aggregateport load-balance src-dst-ip

ip address 10.10.10.254 255.255.255.0

!

interface AggregatePort 10

switchport mode trunk

web-auth enable eportalv2

!

interface VLAN 1

ip address 172.16.1.1 255.255.255.0

!

interface VLAN 10

ip address 172.16.10.1 255.255.252.0

!

interface VLAN 20

ip address 172.16.20.1 255.255.255.0

!

interface Mgmt 0

ip address 192.168.1.200 255.255.255.0

!

ip route 0.0.0.0 0.0.0.0 AggregatePort 1 10.10.10.1

!

no offline-detect

station-move permit

!

line console 0

line vty 0 4

password ruijie123

!

end

NBS5750-E#

### 1 配置文件参考2（5750C)

[BEGIN] 2018/4/23 16:13:09

no zam

!

vlan range 1,10

!

interface GigabitEthernet 0/1

no switchport

ip address 10.10.10.2 255.255.255.0

!

interface GigabitEthernet 0/2

switchport access vlan 10

!

interface GigabitEthernet 0/3

switchport access vlan 10

web-auth enable eportalv2

!

interface GigabitEthernet 0/4

!

interface GigabitEthernet 0/5

!

interface GigabitEthernet 0/6

!

interface GigabitEthernet 0/7

!

interface GigabitEthernet 0/8

!

interface GigabitEthernet 0/9

!

interface GigabitEthernet 0/10

!

interface GigabitEthernet 0/11

!

interface GigabitEthernet 0/12

!

interface GigabitEthernet 0/13

!

interface GigabitEthernet 0/14

!

interface GigabitEthernet 0/15

!

interface GigabitEthernet 0/16

!

interface GigabitEthernet 0/17

!

interface GigabitEthernet 0/18

!

interface GigabitEthernet 0/19

!

interface GigabitEthernet 0/20

!

interface GigabitEthernet 0/21

!

interface GigabitEthernet 0/22

!

interface GigabitEthernet 0/23

!

interface GigabitEthernet 0/24

!

interface GigabitEthernet 0/25

!

interface GigabitEthernet 0/26

!

interface GigabitEthernet 0/27

!

interface GigabitEthernet 0/28

!

interface TenGigabitEthernet 0/29

!

interface TenGigabitEthernet 0/30

!

interface TenGigabitEthernet 0/31

!

interface TenGigabitEthernet 0/32

!

interface VLAN 10

ip address 192.168.10.1 255.255.255.0

!

interface Mgmt 0

!

ip route 0.0.0.0 0.0.0.0 10.10.10.1

!

line console 0

line vty 0 4

!

end

Ruijie(config)#end

*Apr 23 16:12:22: %SYS-5-CONFIG_I: Configured from console by console

Ruijie#show run

Ruijie#show running-config

Building configuration...

Current configuration: 2208 bytes

version S5700H_RGOS 11.4(1)B12P8

!

no spanning-tree

!

http redirect direct-site 10.10.10.1

http redirect direct-site 192.168.10.1 arp

!

web-auth template eportalv2

ip 47.93.25.25

url http://47.93.25.25:9090

fmt cmcc-ext1

!

web-auth portal key natshell

!

aaa new-model

!

aaa accounting update

aaa accounting network default start-stop group radius

aaa authentication web-auth default group radius

!

cwmp

!

service dhcp

!

ip dhcp pool 11

network 192.168.10.0 255.255.255.0

dns-server 202.99.166.4

default-router 192.168.10.1

!

install 0 S5750C-28SFP4XS-H

!

sysmac 0074.9c29.3372

!

mmu buffer-mode normal

!

nfpp

!

no service password-encryption

!

radius-server host 47.93.25.25 key natshell

!

redundancy

!

no zam

!

vlan range 1,10

!

interface GigabitEthernet 0/1

no switchport

ip address 10.10.10.2 255.255.255.0

!

interface GigabitEthernet 0/2

switchport access vlan 10

!

interface GigabitEthernet 0/3

switchport access vlan 10

web-auth enable eportalv2

!

interface GigabitEthernet 0/4

!

interface GigabitEthernet 0/5

!

interface GigabitEthernet 0/6

!

interface GigabitEthernet 0/7

!

interface GigabitEthernet 0/8

!

interface GigabitEthernet 0/9

!

interface GigabitEthernet 0/10

!

interface GigabitEthernet 0/11

!

interface GigabitEthernet 0/12

!

interface GigabitEthernet 0/13

!

interface GigabitEthernet 0/14

!

interface GigabitEthernet 0/15

!

interface GigabitEthernet 0/16

!

interface GigabitEthernet 0/17

!

interface GigabitEthernet 0/18

!

interface GigabitEthernet 0/19

!

interface GigabitEthernet 0/20

!

interface GigabitEthernet 0/21

!

interface GigabitEthernet 0/22

!

interface GigabitEthernet 0/23

!

interface GigabitEthernet 0/24

!

interface GigabitEthernet 0/25

!

interface GigabitEthernet 0/26

!

interface GigabitEthernet 0/27

!

interface GigabitEthernet 0/28

!

interface TenGigabitEthernet 0/29

!

interface TenGigabitEthernet 0/30

!

interface TenGigabitEthernet 0/31

!

interface TenGigabitEthernet 0/32

!

interface VLAN 10

ip address 192.168.10.1 255.255.255.0

!

interface Mgmt 0

!

ip route 0.0.0.0 0.0.0.0 10.10.10.1

!

line console 0

line vty 0 4

!

end

Ruijie#show ver

System description      : Ruijie 10G Ethernet Switch(S5750C-28SFP4XS-H) By Ruijie Networks

System start time       : 2018-04-23 15:16:53

System uptime           : 0:00:55:38

System hardware version : 1.01

System software version : S5700H_RGOS 11.4(1)B12P8

System patch number     : NA

System serial number    : G1MV145000038

System boot version     : 1.2.28

Module information:

Slot 0 : S5750C-28SFP4XS-H

Cpu 0:

Hardware version    : 1.01

Boot version        : 1.2

Software version    : S5700H_RGOS 11.4(1)B12P8

Serial number       : G1MV145000038

[END] 2018/4/23 16:13:35